INTRODUCTION: -

Classical signal processing methods have been used for decades to solve data processing problems. Those methods are model based and, as every model is imperfect, they potentially exhibit a high bias. However, such methods benefit from a relatively low complexity. Recently, machine learning methods have introduced a paradigm shift: rather than using models, one can use generic neural architectures that learn from data. Such methods exhibit a low bias due to their intrinsic adaptability, but the computational and sample complexity of their training can be very high. Model-based machine learning [1] proposes to take the best of both worlds: achieving at the same time low bias and lowcomplexity by using models from signal processing to initialize, structure, and train learning methods. Thefield ofcommunication engineering is particularly well adaptedtotheuseofmodel-based(MB)machinelearningasmany models have been developed to describe communication systems. More specifically, one can use propagation channel models to achieve accurate channel estimation [2, 3, 4, 5]. As propagation channels are highly correlated to the user’s location, one could learn the location-to-channel mapping whose knowledge could be useful in many applications. Indeed, it makes location-based 979-8-3503-4485-1/24/$31.00 ©2024 IEEE channel estimation possible, but also radio-environments com pression: the radio environment around a base station (BS) would be stored in the weights and biases of the trained neural net work. Apart from that, it could also serve beamformer prediction, jammingdetection, resource allocation or even secure commu nication mechanisms. However, the spatial dependence of the underlying model varies on the order of the wavelength, making this mapping remarkably complex to learn. As a matter of fact, it hasbeenprovenin[6]thatclassicalneuralarchitecturestendtobe biased towards learning low frequency functions (spectral bias). Contributions. In this paper, a physics-based channel model is used to derive a model-based neural architecture aimed at learning the location-to-channel mapping in a supervised manner. Similarly to architectures from the implicit neural representation (INR) literature [7, 8, 9] , the proposed neural network presents a spectral separation stage that splits the low frequency from the high frequency content of the target mapping function. This allows to bypass the spectral bias issue. However, in opposition to INRarchitectures, the proposed architecture has an additional hypernetwork whose role is to learn activation coefficients of the high frequency components in a sparse manner. The proposed architecture is evaluated on realistic synthetic data against classical and INR neural architectures. It yields a huge improvement in reconstruction performances but also a drastic reduction in the number of parameters to learn. Related work. Learning mappings through neural networks has been extensively studied in the INR community for image reconstruction [8, 10, 11] and 3D scene reconstruction from 2D images[9,12,13]. Aspecificfocushasbeenpaidonfindingarchi tectures that can learn high frequency details [7, 8, 9]. Moreover, using machine learning methods in order to achieve channel esti mation has attracted a huge interest in the past years [14, 15, 16]. Previous works about the learning of the location or pseudo location to beamformer mapping exist [17, 18]. However, to the best of the authors’ knowledge, there is no previous work specifically focused on learning the location-to-channel mapping.